# AEGIS — User Test Protocol

AI Enhanced Guidance and Inspection System · kitting error detection for TE Connectivity

One operator per session · one cell · no simultaneous testing · target 30 min – 1 h. This document is read and filled top-to-bottom in the order the session runs: setup → briefing → warm-up → Block 1 → reconfigure → Block 2 → debrief. Reference material (BOM order, requirements traceability, per-operator summary) sits at the end.

Filling note: blanks and response cells are click-to-edit after uploading to Google Docs. For questions with options in brackets (e.g. Yes / Partly / No), write or circle the answer. Fields marked [live] are logged in real time during the run; [video] fields are reconciled afterward against the room-camera footage.

## 0. Before the operator arrives (facilitator — not read aloud)

Session identity & assignment

| Field | Value |
| --- | --- |
| Operator Name / ID | Chloe |
| Date | 8 July |
| Facilitator |  |
| Group (counterbalancing — assign ~half to each): A — Control block first, System second · B — System block first, Control second | B |
| Randomised Control BOM order | 231 |
| Randomised System BOM order | 231 |

Setup checklist

- ☐ Cell assembled; 9 bins staged per the fixed layout (see reference at end), filled/topped to their baselines.

- ☐ Camera height / x / y / layers set to the dev-test locked operating point.

- ☐ Lighting set to the dev-test locked W / lux; lux meter confirms.

- ☐ Dashboard up (`localhost:8080`); calibrate, init kit; all 9 bins detected and reading sane counts.

- ☐ Load cells tared with baselines in place; per-bin unit weights loaded.

- ☐ Paper BOM / sub-kit build sheets printed for the Control block; answer keys (part → required qty per set) printed for facilitator inspection.

- ☐ HMI ready for the System block.

- ☐ Stopwatch ready; a larger kit container ready to receive emptied sub-kits.

- ☐ Room camera positioned to capture bins + station + screen + operator; recording ready.

- ☐ This protocol + the debrief printed / open; consent (Section 1) ready to fill.

## 1. Briefing (read aloud to the operator)

Square-bracketed `[…]` text is a stage direction, not read aloud.

### 1.1 Welcome & problem context

"Thanks for coming in, and thanks for helping us test our system. Before we start I'll walk you through what this is about, how the system works, what you'll be doing, and I'll ask your permission to record. Stop me any time with questions.

We're building a system for manual kitting — the job of picking the right parts, in the right quantities, and packing them into a kit that goes down the line. This is done by hand at assembly workstations, and the people doing it are the operators — that's the role you'll be playing today. The problem owner is TE Connectivity, a company that assembles electronic and electrical components, and this is a final-year engineering capstone project.

The reason this matters: when a kit goes out with the wrong part or the wrong quantity, that error travels downstream. It can stop a line, force rework, or reach a customer — and the further it travels before someone catches it, the more expensive it gets. So the goal of our system is simple: catch kitting mistakes at the workstation, the moment they happen, before the kit leaves."

### 1.2 What the system is and how it works

"Here's how our system — we call it AEGIS — works.

There's an overhead camera above the bin rack that watches the bins in real time. It sends what it sees to a computer. Under each bin there's also a load cell — a weight sensor — so the computer knows, by weight, how many parts have been taken from each bin. The computer combines the camera view and the weight readings to track your kitting progress and your actions as you work.

On the screen in front of you, the system shows you:

- The parts and quantities you need to pack for the current set — your on-screen build list.

- Which bins are still incomplete — you pick from one of those.

- A running count of how many you've packed from each bin, so you can always see where you are.

- Warnings and errors, if something goes wrong — I'll explain those in a second.

- Corrective actions — if you make a mistake, it tells you exactly what to do to fix it, for example 'return one part to bin 3.'

There are two levels of alert:

- A warning is minor. If you overpick — take one more of a part than the set needs — you'll see a small red indicator. It's not fatal. You can carry on and pack, as long as you don't make a serious error. It's just letting you know.

- An error is serious and shows as a full red screen. This happens if you pick from the wrong bin, if you overpack a set, or if you return a part to the wrong bin. When you see a red screen, you must fix it before you can carry on — the system won't let you move forward until it's resolved. Just follow the corrective instruction on the screen."

### 1.3 How you'll work — the sets and the buttons

"An order isn't one big pack — it's broken into sets, one after another. Think of each set as a sub-kit: a small group of parts that gets packed, checked, and set aside, before you start the next one. A single order can have several sets.

In the system block, here's the rhythm for each set:

- The screen shows you the parts and quantities for the current set.

- You pack them, picking from the bins.

- When the set is done and correct, a 'Complete Set' button appears — press it. Note: this button will only appear once the set is correct. If anything's missing, over-packed, under-packed, or wrong, the button won't show up — so if you can't find it, check the screen for what still needs fixing.

- After you press 'Complete Set', empty the packed parts out of the kitting bin into the larger kit container — you're consolidating that sub-kit into the finished kit.

- Then press the 'Bin Emptied' button to tell the system the sub-kit is out and you're ready for the next set.

- The next set's build list appears, and you repeat — until the whole order is done.

One working rule for the system block: as a standard operating procedure, please pick from one bin at a time — finish a bin's full required quantity before you move on to the next bin. The system doesn't force this and doesn't track it, but it's how we'd like you to work in this block. You still choose which bin to do first — just finish the one you're on before starting another.

In the control block, it's the same idea but without the screen. You'll work from a paper build sheet instead. For each set: pack the parts listed, and when you believe the set is done, tell me 'set complete' — I'll check it. Then, just like with the system, empty the parts out into the kit container to simulate consolidating the sub-kit, and I'll tell you when to start the next set. In the control block you can pick in any order you like — there's no on-screen rule and no feedback; you just build from the sheet and I inspect afterward."

### 1.4 What we're going to do today

"Here's the plan for the session:

- First, a warm-up: you'll build all three orders once, just to get familiar with the parts and the flow. Nothing in the warm-up is measured — it's only so a new order isn't landing on you cold during the real runs.

- Then two blocks of real runs. In one block you'll work without the system — the control — building three orders from paper. In the other block you'll work with the system — building the same three orders using the screen.

- Between the two blocks I'll take a moment to switch the system on or off and re-check the setup.

- At the very end there's a short part where I'll ask you to deliberately trigger a few of the error states, so you can see how each alert looks and tell us what you think of it.

- Finally, a short interview about how it felt to use.

A few things to keep in mind:

- Work at a natural pace and a natural level of care — there's no race and no clock you need to beat. Just work the way you normally would.

- It's completely fine to make mistakes. We're testing the system, not you. Mistakes are useful data for us — don't worry about them.

- I'll be timing and taking notes while you work. If I'm quiet, that's why — just carry on.

- The whole session takes about 30 minutes to an hour."

### 1.5 Consent to record (read aloud, then fill in)

"Before we begin I need your permission to record video. Here's exactly how it's used:

- We record the workstation with a room camera — it captures the bins, the station, the screen, and you, so we can review what happened afterward.

- The footage is used only for our own analysis by the project team.

- It may be included in our final report, which goes to our school and our course coordinators.

- It will not be given to any outside organisation — including Advantech and TE Connectivity.

- The footage is kept only for the duration of the project.

- You can stop or withdraw at any point — just say so, and we'll stop.

Do I have your permission to record video on those terms?"

- Consent to video recording (Y / N):

[If No:] "That's completely fine." [Proceed without video; rely on live facilitator logging only.]

### 1.6 Final check

"Any questions before we start? [Answer any questions.] Great — we'll begin with the warm-up. I'll show you the first order; take your time, and remember nothing here is being measured yet."

## 2. Warm-up (un-logged)

- ☐ Operator builds all 3 BOMs once, un-logged, to familiarise with parts and flow. Not measured. Use paper or the mode of whichever block comes first for this operator.

## 3. Block 1

Cycle-time definition (both blocks): start = the BOM is presented (System: build list flashes on screen / Control: facilitator says "go" and hands the sheet). end = operator presses 'Bin Emptied' after the last set (System) / declares the last sub-kit emptied (Control). Timing is recorded manually against the footage. The clock keeps running through in-process error corrections.

Flag / event log legend (System runs):

- Type: `overpick-warn` (minor) · `wrong-bin-not-in-set` · `wrong-bin-completed` · `overpack` · `wrong-return-bin` · `underpack-block` · `other`

- Correct? did the system's flag match what actually happened? Y (system right) · N (system wrong — false flag / missed / misclassified) · NA

- Resolved? did the operator clear it before proceeding? Y / N

Block 1 is (Control = Group A / System = Group B): _____ — fill to match this operator's group. Fill the same run-sheet fields either way; the run type at the top of each run records which mode applied.

### Run 1.1

Run header

| Field | Value |
| --- | --- |
| Run type (Control / System) |  |
| BOM (1 / 2 / 3) |  |
| [video] Start (BOM presented) |  |
| [video] End (last sub-kit emptied / 'Bin Emptied') |  |
| [video] Whole-order cycle time (s) |  |

Per-set timing (reconciled from video)

| Set | Start | End | Set time (s) | Notes |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Flag / event log (System runs only; [live] rough, complete from [video])

| # | Set | Timestamp | Type | Description | Correct? (Y/N/NA) | Resolved? (Y/N) | Time-to-resolve (s) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |

Facilitator inspection after each set

| Set | 'Complete Set' allowed by system? (Y/N/n.a., System only) | Inspection vs answer key (correct/defect) | Escaped defects (count) | Error type(s) | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

- Whole-order defect-free? (Y / N): · Total escaped defects: · In-process catches (System):

- System anomalies (false flag / missed catch / freeze / UI confusion / none):

- Notes:

### Run 1.2

Run header

| Field | Value |
| --- | --- |
| Run type (Control / System) |  |
| BOM (1 / 2 / 3) |  |
| [video] Start (BOM presented) |  |
| [video] End (last sub-kit emptied / 'Bin Emptied') |  |
| [video] Whole-order cycle time (s) |  |

Per-set timing (reconciled from video)

| Set | Start | End | Set time (s) | Notes |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Flag / event log (System runs only; [live] rough, complete from [video])

| # | Set | Timestamp | Type | Description | Correct? (Y/N/NA) | Resolved? (Y/N) | Time-to-resolve (s) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |

Facilitator inspection after each set

| Set | 'Complete Set' allowed by system? (Y/N/n.a., System only) | Inspection vs answer key (correct/defect) | Escaped defects (count) | Error type(s) | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

- Whole-order defect-free? (Y / N): · Total escaped defects: · In-process catches (System):

- System anomalies (false flag / missed catch / freeze / UI confusion / none):

- Notes:

### Run 1.3

Run header

| Field | Value |
| --- | --- |
| Run type (Control / System) |  |
| BOM (1 / 2 / 3) |  |
| [video] Start (BOM presented) |  |
| [video] End (last sub-kit emptied / 'Bin Emptied') |  |
| [video] Whole-order cycle time (s) |  |

Per-set timing (reconciled from video)

| Set | Start | End | Set time (s) | Notes |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Flag / event log (System runs only; [live] rough, complete from [video])

| # | Set | Timestamp | Type | Description | Correct? (Y/N/NA) | Resolved? (Y/N) | Time-to-resolve (s) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |

Facilitator inspection after each set

| Set | 'Complete Set' allowed by system? (Y/N/n.a., System only) | Inspection vs answer key (correct/defect) | Escaped defects (count) | Error type(s) | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

- Whole-order defect-free? (Y / N): · Total escaped defects: · In-process catches (System):

- System anomalies (false flag / missed catch / freeze / UI confusion / none):

- Notes:

## 4. Reconfigure (between blocks)

- ☐ Toggle AEGIS on/off for the second block; recalibrate; re-confirm all 9 bins detected. (The single costly switch.)

## 5. Block 2

Block 2 is (System = Group A / Control = Group B): _____ — the opposite of Block 1. Fill the run type on each run sheet to match.

### Run 2.1

Run header

| Field | Value |
| --- | --- |
| Run type (Control / System) |  |
| BOM (1 / 2 / 3) |  |
| [video] Start (BOM presented) |  |
| [video] End (last sub-kit emptied / 'Bin Emptied') |  |
| [video] Whole-order cycle time (s) |  |

Per-set timing (reconciled from video)

| Set | Start | End | Set time (s) | Notes |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Flag / event log (System runs only; [live] rough, complete from [video])

| # | Set | Timestamp | Type | Description | Correct? (Y/N/NA) | Resolved? (Y/N) | Time-to-resolve (s) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |

Facilitator inspection after each set

| Set | 'Complete Set' allowed by system? (Y/N/n.a., System only) | Inspection vs answer key (correct/defect) | Escaped defects (count) | Error type(s) | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

- Whole-order defect-free? (Y / N): · Total escaped defects: · In-process catches (System):

- System anomalies (false flag / missed catch / freeze / UI confusion / none):

- Notes:

### Run 2.2

Run header

| Field | Value |
| --- | --- |
| Run type (Control / System) |  |
| BOM (1 / 2 / 3) |  |
| [video] Start (BOM presented) |  |
| [video] End (last sub-kit emptied / 'Bin Emptied') |  |
| [video] Whole-order cycle time (s) |  |

Per-set timing (reconciled from video)

| Set | Start | End | Set time (s) | Notes |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Flag / event log (System runs only; [live] rough, complete from [video])

| # | Set | Timestamp | Type | Description | Correct? (Y/N/NA) | Resolved? (Y/N) | Time-to-resolve (s) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |

Facilitator inspection after each set

| Set | 'Complete Set' allowed by system? (Y/N/n.a., System only) | Inspection vs answer key (correct/defect) | Escaped defects (count) | Error type(s) | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

- Whole-order defect-free? (Y / N): · Total escaped defects: · In-process catches (System):

- System anomalies (false flag / missed catch / freeze / UI confusion / none):

- Notes:

### Run 2.3

Run header

| Field | Value |
| --- | --- |
| Run type (Control / System) |  |
| BOM (1 / 2 / 3) |  |
| [video] Start (BOM presented) |  |
| [video] End (last sub-kit emptied / 'Bin Emptied') |  |
| [video] Whole-order cycle time (s) |  |

Per-set timing (reconciled from video)

| Set | Start | End | Set time (s) | Notes |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

Flag / event log (System runs only; [live] rough, complete from [video])

| # | Set | Timestamp | Type | Description | Correct? (Y/N/NA) | Resolved? (Y/N) | Time-to-resolve (s) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |  |  |  |
| 2 |  |  |  |  |  |  |  |  |
| 3 |  |  |  |  |  |  |  |  |
| 4 |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |

Facilitator inspection after each set

| Set | 'Complete Set' allowed by system? (Y/N/n.a., System only) | Inspection vs answer key (correct/defect) | Escaped defects (count) | Error type(s) | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 5 |  |  |  |  |  |

- Whole-order defect-free? (Y / N): · Total escaped defects: · In-process catches (System):

- System anomalies (false flag / missed catch / freeze / UI confusion / none):

- Notes:

## 6. Debrief — Part 1: Error-state walkthrough (system on)

[Read aloud:] "Now that you've built the orders, I'd like to show you the system's alerts on purpose. I'll ask you to deliberately make a few mistakes, one at a time, so you can see how each alert looks and tell me what you think of it. Nothing here is a test of you — I'm asking you to make these errors."

For each state: [prompt the operator to trigger it], let them see the alert, ask the questions, record. Reset before the next.

### 6.1 Overpick warning (minor — small red indicator)

[Prompt:] "Take one more of this part than the set asks for."

- Was it clear what this alert was telling you? (Clear / Unclear):

- What did you understand it wanted you to do (if anything)?

- Notes:

  - return too small

  - sound or indication for perioheral

### 6.2 Wrong-bin pick — case (a): bin not in the set/BOM

[Prompt:] "Pick a part from a bin that isn't part of this set at all."

- Was it clear this was a serious error (vs. the minor warning)? (Clear / Unclear):

- Did the corrective instruction tell you exactly how to fix it? (Yes / Partly / No):

- Notes:

  - lag, not fast enuf

### 6.3 Wrong-bin pick — case (b): bin already completed in the set

[Prompt:] "Now pick from a bin you've already finished for this set."

- Did the alert make sense given the bin was already done? (Clear / Unclear):

- Was the fix obvious? (Yes / Partly / No):

- Notes:

  - too wordy, banner should be in the middle of the screen

### 6.4 Overpack (fatal — full red screen)

[Prompt:] "Pack more of a required part than the set needs, past the warning, into the kit."

- Did you understand the difference between this red-screen error and the earlier small overpick warning? (Yes / Somewhat / No):

- Notes:

  - same comments

### 6.5 Wrong-bin return (fatal — full red screen)

[Prompt:] "When returning a part, put it back into the wrong bin."

- Was it clear what had gone wrong and how to correct it? (Clear / Unclear):

- Notes:

  - same comments

### 6.6 Underpack / missing-item block on 'Complete Set'

[Prompt:] "Leave one required part short, then try to complete the set."

- Did you understand why the 'Complete Set' button wouldn't appear? (Yes / Somewhat / No):

- Did the screen make it clear what was still missing? (Yes / Partly / No):

- Notes:

  - assume computer at fault

Overall on the alerts (open):

- Which alert was the clearest? Which was the most confusing?

  - X is the clearest (hand in wrong bin)

  - don’t see the diff between diff errors, all look similar

  - bigger signals for more severe errors

- Did any alert feel unnecessary or annoying?

- If you could reword or redesign one alert, which and how?

## 7. Debrief — Part 2: Qualitative interview

[Read aloud:] "Last part — just a short interview about how it felt. For some of these I'll ask you to give a rating from 1 to 5, then tell me why. Be honest — critical feedback is the most useful thing you can give us."

### 7.1 Ratings (1 = poor / strongly disagree, 5 = excellent / strongly agree)

Record the number, then always ask "why that number?" and capture the reason — the why matters more than the score.

| # | Dimension | Rating (1–5) | Why? (capture verbatim) |
| --- | --- | --- | --- |
| R1 | The wording on the screen was clear and easy to understand | 2 | small, coding font |
| R2 | The placement / layout of information on the screen was easy to follow | 4 | can put screen in front |
| R3 | It was easy to understand what to pack | 4.5 | sometime forget the bottom row refers to the bottom layer |
| R4 | It was easy to understand how many to pack | 5 |  |
| R5 | The running count per bin helped me keep track | 1 | lag, more efficient to just count again |
| R6 | The warnings and error alerts were easy to understand | 2 | all look similar, but know its an error |
| R7 | The corrective instructions told me clearly how to fix mistakes | 2 | same comments |
| R8 | Overall, I would trust this system to catch my mistakes | 3 | effective but inefficient |
| R9 | Overall, the system made kitting easier than working without it | 4 |  |

### 7.2 Open questions

Usability — convenience of the process

- Did the empty-out / 'Bin Emptied' step between sets feel natural, or like a chore?

  - satisfying, confirmation of set completion. bin is heavy

Cognitive load — getting at the root cause (find out not just whether it felt heavy, but why )

- Across the whole task, when did you feel most mentally loaded or stressed — and what specifically caused it? (the counting? the alerts? the screen? the pace? finding parts?)

  - hard to take shelf bracket

  - system tell you which system is most optimal. i.e. bulky items should go last since it is harder to handle

- Did the alerts interrupt your flow? When one popped up, did you lose your place or have to restart a train of thought?

  - no, cos its at the side. diff story if in front

- If one single thing made the task feel busier or more tiring than it needed to be, what was it?

  - waiting

Wrap-up

- Anything else you noticed that we didn't ask about?

  - no

### 7.3 Facilitator post-session notes (fill after the operator leaves)

- Standout usability problems observed (not just reported):

- Any system anomalies not yet logged (false flag / missed catch / freeze / UI confusion):

- Did control-vs-system order (Group A/B) seem to affect this operator?

- Candidate v2 backlog items surfaced:

# Reference (not part of the live run)

## R-1. Per-operator summary (fill after all 6 runs)

Raw numbers only — computed comparisons happen post-session across operators. No pass/fail lines.

| BOM | Control cycle time (s) | System cycle time (s) | Control escaped defects | System escaped defects | System in-process catches | System anomalies |
| --- | --- | --- | --- | --- | --- | --- |
| 1 (easy) |  |  |  |  |  |  |
| 2 (medium) |  |  |  |  |  |  |
| 3 (hard) |  |  |  |  |  |  |

## R-2. Fixed bin layout (current — BOM v3)

The 9 instrumented bins hold fixed contents for the whole session (no re-staging between orders). Full per-set BOM contents live in the separate BOM reference / Mock BOM files; this table is the bin→part map only.

| Bin | Class / load cell | Component | Unit mass |
| --- | --- | --- | --- |
| S1 | Small · 1 kg single-cell | TE Case | 3.5 g |
| S2 | Small · 1 kg single-cell | M6 Cross Screw | 3.5 g |
| S3 | Small · 1 kg single-cell | Shelf Bracket | 5.0 g |
| M1 | Medium · 5 kg 3-cell | Bin Connector | 13.0 g |
| M2 | Medium · 5 kg 3-cell | M10 CS Allen Screw | 45.0 g |
| M3 | Medium · 5 kg 3-cell | M8 Allen Pan Screw | 27.7 g |
| L1 | Large · 10 kg 3-cell | Copper Rod | 27.5 g |
| L2 | Large · 10 kg 3-cell | M12 Hex Bolt | 74.5 g |
| L3 | Large · 10 kg 3-cell | PVC 3-Way Connector | 130 g |

Known limitation: the 3-cell bins (Medium and Large) carry the off-center sensitivity documented in the load-cell qualification; accepted as-is for this test.

## R-3. Requirements traceability (what this user test evidences)

Ergonomics is out of scope (Demand 1 dropped; Demands 2–3 only partially touched via on-screen part/quantity guidance). Computed figures only — no pass/fail judgement here.

| Measure captured here | Requirement it informs |
| --- | --- |
| Cycle time, control vs system, per BOM | Demand 10 (cycle-time impact vs baseline) |
| In-process catches; escaped defects control vs system | Demands 6–7, 9 (per-pick identity/quantity verification; immediate feedback & correction) |
| Flag correctness (Correct? column) — indicative only | Demand 8 (verification accuracy) — rigorous TP/TN/FP/FN rates measured in the developer test, not here |
| Debrief ratings R1–R7; open usability & cognitive-load questions | Wishes 2, 8 (visual guidance clarity; operator-friendly interaction) |
| Debrief trust/ease ratings R8–R9; "would you want this" | Wishes 8–9 (adoption, operator-facing usability) |

Classification-accuracy rates (true/false positive & negative) are deliberately measured in the developer test's scripted error battery, where the error denominator is known. This user test measures whether the system catches naturally-occurring operator errors and how usable/trusted it is.